NORTHERN ILLINOIS UNIVERSITY

PHYSICS DEPARTMENT

Physics 374 – Junior Physics Lab					Spring 2021

Python Tutorial #6

Python Least Squares Fitting

In this tutorial, we will learn how to how to use the least squares fitting routines in the scipy module to do nonlinear fits to data.  To use scipy, we must install its module using pip (see Python Tutorial #1)—open up Windows PowerShell and type:

py -m pip install scipy

A good tutorial for scipy can be found at:

https://python4mpia.github.io/fitting_data/least-squares-fitting.html and https://docs.scipy.org/doc/scipy-0.15.1/reference/generated/scipy.optimize.curve_fit.html.

We will use the curve_fit function in the scipy module to fit functions.  It applies the Levenburg-Marquardt gradient method using what is called a greedy algorithm to minimize chi-squared.

To introduce us to scipy, let’s fit the data in Bevington, Table 7.2, with a straight line.  To use the curve_fit function in the scipy module, we must import scipy into to the program:

import scipy.optimize as opt

In the code below, the six voltage and temperature data points are contained in the xdata and ydata arrays.  Since this is a numerical least squares fitting method, we have to provide initial guesses of the parameter values.  The guesses of the parameters are contained in the array called guess.  If the initial guesses are far from the optimal parameter values that miminize , then the numerical calculations may never converge to the correct solutions.  Thus, it is important to make good initial guesses for the parameters.  The array sigma contains the uncertainties of the measured values (in this case, the voltages).  Using the numpy routine fill, we can fill the array sigma with equal uncertainties of 0.05 volts for all measurements.  The function calfun(x, a1, a2) contains the curve we wish to fit the data to, which, in this case, is the equation of a straight line.  The least squares fitting routine, curve_fit, applies the Levenburg-Marquardt gradient technique to fit the data.  The output from curve_fit is the final parameter file par and the error (or covariance) matrix error.  Open up Microsoft Visual Studio and copy and paste the following code into a project called Least_Squares:

import numpy as np  # the alias for "numpy" will be "np"

import matplotlib.pyplot as plt  # the alias for "matplotlib.pyplot" will be "plt"

import scipy.optimize as opt  # the alias for "scipy.optimize" will be "opt"

xdata = np.array([0.0, 20.0, 40.0, 60.0, 80.0, 100.0])  # Temperature data

ydata = np.array([-0.849, -0.196, 0.734, 1.541, 2.456, 3.318])  # Voltage data

guess = np.array([0.01, -0.1])  # initial guesses for parameters a1 & a2

# for the linear fit y = a1 + a2*x

sigma = np.empty([6])  # create an empty array of six elements

sigma.fill(0.05)  # uncertainties all voltages measurements are 0.05 volts

# "fill" sets all elements of the array to be the same number

def calfun(x, a1, a2):  # defining the function y = a1 + a2*x

return a1 + a2*x

#

# par   --> contains the final parameter values of the fit

# error --> contains the error, or covariance, matrix

# absolute_sigma=False --> Sigma contains relative weights of the data points.

#                          The covariance matrix will be based on estimated

#                          errors in the data

# absolute_sigma=True --> Sigma contains standard deviation errors of the data

#                         points.  The covariance matrix will be based on these

#                         values.

#

par,error = opt.curve_fit(calfun, xdata, ydata, guess, sigma, absolute_sigma=True)

uncertainties = np.sqrt(np.diagonal(error))  # take square root of diagonal matrix

#

#  Printing outputs

#

print(par)  # print the parameter array      # elements

print()

print(error)  # print the error (covariance) matrix

print()

print(uncertainties)  # print the uncertainties of each parameter

print() # "\u00B1" is the +/- symbol  ; ":.8f" = write result to 8 decimal places

print("parameter a1 = {:.8f} \u00B1 {:.8f}".format(par[0],uncertainties[0]))

print("parameter a2 = {:.8f} \u00B1 {:.8f}".format(par[1],uncertainties[1]))

#

#  Plotting outputs

#

plt.plot(xdata,ydata,"ko",label="data",ms=2)  # make a scatter plot for the data

a1, a2 = par  # set a1 and a2 parameters      # "ms" sets the size of the circle

x = np.linspace(min(xdata), max(xdata), 100)  # array of 100 x-axis points

yfit = calfun(x,a1,a2)  # array of yfit values for each value of x array

plt.plot(x,yfit,"--",label="fit")  # plot the fitting curve

plt.errorbar(xdata,ydata,yerr=sigma,capsize=3,fmt='ko',ms=1)  # plot error bars

plt.legend()  # show the legend

plt.title("Least Squares Linear Fit")  # plot title of the plot

# "\N{DEGREE SIGN}" is the degee symbol for degrees Celcius

plt.xlabel("Temperature  (\N{DEGREE SIGN}C)")  # plot the x-axis label

plt.ylabel("Voltage  (V)") # plot the y-axis label

plt.show()  # command to draw the plot on the screen

The output is shown below:

Notice that the parameter array and the error matrix agree with Bevington’s results in Table 7.2 (page 125).

Homework

- Open up your Excel (or OpenDoc) spreadsheet and perform a linear fit to the voltage data in the program above using the LINEST function [if you forgot how to use it, there is a “Linest” link on the Physics 374 WebPage (www.niu.edu/brown)].  You should get the following:

Notice that the parameters, and  from the Python code agree with the LINEST results, but the uncertainties do not agree.  To understand why, let us go back to Eq. (7.11) in Bevington (but I will write it correctly):  The standard deviation squared is defined as

Let the variances of all measurements be the same:

Then,

The reduced chi-squared, , is defined as chi-squared divided by the number of degrees of freedom, , where  is the number of data points minus the number of parameters:

when the variances for all measurements are equal.  Notice that if we set  to  in the expression for , then .  Thus, adjusting the value of the uncertainty in the measurements to make  gives us a way to estimate the variance in the measurements:

or,

Thus,  is the estimated uncertainty in the measurements that will make  (when all variances are equal).

- Calculate  for your data in Problem 1 and use it for the uncertainties in your Python program (instead of  volts).  Show that your results now agree with those of the LINEST results.  That is, LINEST is assuming all uncertainties are equal, and it adjusts that uncertainty in such a way to make the reduced chi-square equal to unity: .

- Write a Python code that fits the thermocouple data in Table 7.1 (page 120) with a quadratic curve  (use the Thermocouple.txt file used in Python Tutorial #5).  Assume all voltages have a common uncertainty of 0.05 V.  Show that your Python code gives the same parameters and uncertainties given in Table 7.1, and that the code gives the same error matrix given in Table 7.3 (page 126).  You do not have to write these results to an output file—just use a snipping tool to take a picture of the Python results (like that shown in this tutorial).

Upload to Blackboard the source code (*.py) for your Python program (do not worry about making executables) and a picture of your results.  You will see an assignment on Blackboard called Python Tutorial #6.