NORTHERN ILLINOIS UNIVERSITY

PHYSICS DEPARTMENT

Physics 374 – Junior Physics Lab					Spring 2026

Python Tutorial #4

Reading and Writing Files

The thermocouple data (voltage verses temperature) is located in the Thermocouple-data-1.txt file (located on the Physics 374 WebPage: www.niu.edu/brown).  Rather than hardcode the file in the program, we would like to have the program prompt us with a file dialog requesting for the file.  This can be done with routines from the tcl/tk modules (see https://docs.python.org/3/library/dialog.html).  To have the program ask for an input data file, use the tkinter function: filedialog.askopenfilename() as shown below:

import numpy as np    # the alias for "numpy" will be "np"

import tkinter as tk            # the alias for "tkinter" will be "tk"

from tkinter import filedialog  # use the filedialog from tkinter

root = tk.Tk()    # shows the root window

root.withdraw()   # hides the root window

inputfile = filedialog.askopenfilename()   # opens a file dialog

print(inputfile)   # prints the file path to the console

x, y = np.loadtxt(inputfile, unpack=True)  # unpack=True transposes columns

print(x)                                   # x = Temperature data points

print(y)                                   # y = Voltage data points

Notice that we have loaded the columns of voltage and temperature in the data file called inputfile into the x and y arrays using np.loadtxt(inputfile, unpack=True).

Now we need a way to save data to an output file.  We can use the filedialog.asksaveasfile() on the python.org webpage to write data into a file called outputfile (https://docs.python.org/3/library/dialog.html).  The following code writes the matrix elements of A to an output file named by the user of the program.

import numpy as np    # the alias for "numpy" will be "np"

import tkinter as tk  # the alias for "tkinter" will be "tk"

from tkinter import filedialog  # use the filedialog from tkinter

A = np.array([[1, 2, 3],

[4, 5, 6],

[7, 8, 9]])

root = tk.Tk()    # shows the root window

root.withdraw()   # hides the root window

#

# askopenfilename opens a file dialog and creates a file path: inputfile

# the default filetype will be *.txt

# otherwise, one can look at all files *.*

#

inputfile = filedialog.askopenfilename(title="Input File",

filetypes = (("Text files","*.txt*"),

("all files","*.*")))

print(inputfile)   # prints the file path to the console

x, y = np.loadtxt(inputfile, unpack=True)  # unpack=True transposes columns

print(x)                                   # x = Temperature data points

print(y)                                   # y = Voltage data points

#

# asksaveasfile opens a file dialog  and creates a file object: outputfile

# the default filetype will be *.txt

# otherwise, one can look at all files *.*

#

outputfile = filedialog.asksaveasfile(title="Output File",

defaultextension=".txt",

filetypes = (("Text files","*.txt*"),

("all files","*.*")))

np.savetxt(outputfile, A, fmt='%2.4f')  # writes matrix A into output file

# in floating xx.xxxx format

outputfile.close()  # close the output file

Python Nonlinear Least Squares Fitting

In this tutorial, we will learn how to how to use the least squares fitting routines in the scipy module to do nonlinear fits to data.  To use scipy, we must install its module using pip (see Python Tutorial #1)—open up Windows PowerShell and type:

py -m pip install scipy

A good tutorial for scipy can be found at:

https://python4mpia.github.io/fitting_data/least-squares-fitting.html and https://docs.scipy.org/doc/scipy-0.15.1/reference/generated/scipy.optimize.curve_fit.html.

We will use the curve_fit function in the scipy module to fit functions.  It applies the Levenburg-Marquardt gradient method using what is called a greedy algorithm to minimize chi-squared.

To introduce us to scipy, let’s fit the data in Bevington, Table 7.2, with a straight line.  To use the curve_fit function in the scipy module, we must import scipy into to the program:

import scipy.optimize as opt

In the code below, the voltage and temperature data contained in the Thermocouple-data-1.txt file (located on the Physics 374 WebPage) are placed in the xdata and ydata arrays.  Since this is a numerical least squares fitting method, we have to provide initial guesses of the parameter values.  The guesses of the parameters are contained in the array called guess.  If the initial guesses are far from the optimal parameter values that miminize , then the numerical calculations may never converge to the correct solutions.  Thus, it is important to make good initial guesses for the parameters.  The array sigma contains the uncertainties of the measured values (in this case, the voltages).  Using the numpy routine fill, we can fill the array sigma with equal uncertainties of 0.05 volts for all measurements.  The function calfun(x, a1, a2) contains the curve we wish to fit the data to, which, in this case, is the equation of a straight line.  The least squares fitting routine, curve_fit, applies the Levenburg-Marquardt gradient technique to fit the data.  The output from curve_fit is the final parameter file par and the error (or covariance) matrix error.  Open up Microsoft Visual Studio and copy and paste the following code into a project called Least_Squares:

import numpy as np  # the alias for "numpy" will be "np"

import matplotlib.pyplot as plt  # the alias for "matplotlib.pyplot" will be "plt"

import scipy.optimize as opt  # the alias for "scipy.optimize" will be "opt"

import tkinter as tk            # the alias for "tkinter" will be "tk"

from tkinter import filedialog  # use the filedialog from tkinter

root = tk.Tk()    # shows the root window

root.withdraw()   # hides the root window

inputfile = filedialog.askopenfilename()   # opens a file dialog

print(inputfile)   # prints the file path to the console

xdata, ydata = np.loadtxt(inputfile, unpack=True)  # reading data

# xdata = Temperature data points

# ydata = Voltage data points

Npts = len(ydata)

guess = np.array([0.01, -0.1])  # initial guesses for parameters a1 & a2

# for the linear fit y = a1 + a2*x

sigma = np.empty([Npts])  # create an empty array of six elements

sigma.fill(0.05)  # uncertainties all voltages measurements are 0.05 volts

# "fill" sets all elements of the array to be the same number

def calfun(x, a1, a2):  # defining the function y = a1 + a2*x

return a1 + a2*x

#

# par   --> contains the final parameter values of the fit

# error --> contains the error, or covariance, matrix

# absolute_sigma=False --> Sigma contains relative weights of the data points.

#                          The covariance matrix will be based on estimated

#                          errors in the data

# absolute_sigma=True --> Sigma contains standard deviation errors of the data

#                         points.  The covariance matrix will be based on these

#                         values.

#

par,error = opt.curve_fit(calfun, xdata, ydata, guess, sigma, absolute_sigma=True)

uncertainties = np.sqrt(np.diagonal(error))  # take square root of diagonal matrix

#

#  Printing outputs

#

print(par)  # print the parameter array      # elements

print()

print(error)  # print the error (covariance) matrix

print()

print(uncertainties)  # print the uncertainties of each parameter

print() # "\u00B1" is the +/- symbol  ; ":.8f" = write result to 8 decimal places

print("parameter a1 = {:.8f} \u00B1 {:.8f}".format(par[0],uncertainties[0]))

print("parameter a2 = {:.8f} \u00B1 {:.8f}".format(par[1],uncertainties[1]))

#

#  Plotting outputs

#

plt.plot(xdata,ydata,"ko",label="data",ms=2)  # make a scatter plot for the data

a1, a2 = par  # set a1 and a2 parameters      # "ms" sets the size of the circle

x = np.linspace(min(xdata), max(xdata), 100)  # array of 100 x-axis points

yfit = calfun(x,a1,a2)  # array of yfit values for each value of x array

plt.plot(x,yfit,":",label="fit")  # plot the fitting curve with a dotted curve “:”

plt.errorbar(xdata,ydata,yerr=sigma,capsize=3,fmt='ko',ms=1)  # plot error bars

plt.legend()  # show the legend

plt.title("Least Squares Linear Fit")  # plot title of the plot

# "\N{DEGREE SIGN}" is the degee symbol for degrees Celcius

plt.xlabel("Temperature  (\N{DEGREE SIGN}C)")  # plot the x-axis label

plt.ylabel("Voltage  (V)") # plot the y-axis label

plt.show()  # command to draw the plot on the screen

The output is shown below:

Notice that the parameter array and the error matrix agree with Bevington’s results in Table 7.2 (page 125).

Homework

Write the data circled in the figure above into an output file.

Upload to Blackboard the source code (*.py) for your Python program and your output file.  You will see an assignment on Blackboard called Python Tutorial #4.